How to test a charger

- Check voltage between 1 and 2 on the charger socket.

2.  If there is no voltage present:

a. the batteries are not hooked up

b. the circuit breaker is tripped

c. a fuse is blown

d. a break in the wire harness

3.  If there is a voltage reading proceed with testing the battery charger

4.  Take a voltage reading directly from the batteries between points 1 and 2.  Note the voltage.

- Connect the battery charger.

- After 10 minutes of charging and with the charger still connected, repeat step #4.   If the charger is working properly there will be an increase in the voltage.  If the initial voltage was below 22 volts this test will not work properly.